| Datum |  | Vår referens |
| --- | --- | --- |
| 04/05/2016 |  |

SLUTOFFERT

NLX 2500SY | 700 Premium Design

Projekt nr 543384

-------------------------------

NLX 2500SY | 700 Premium Design

Highlights

- Slideways are used for all axes

- Coolant circulation inside the castings controls thermal displacement

- With BMT (Built-in Motor Turret), milling capability is comparable to machining centers (SY type)  - Y-axis travel: ±50 mm (SY type)

- Max. turning length 705 mm (Distance between centers of 700 spec.)

- Spindle 2 max. spindle speed: 6,000 min-1 (SY type)

- New common premium design

GILDEMEISTER ITALIANA NLX 2500SY | 700 Premium Design

Basic machine

| I-A00577* Control | NLX 2500SY ¦ 700 Universal lathe Distance between centers: 700 mm (27.6 in.) Spindle 1: 4,000 min-1, 18.5/18.5/15 kW (24.7/24.7/20 HP) <25%ED/50%ED/cont> Bar work capacity: dia. 80 mm (3.1 in.) Spindle 2: 6,000 min-1, 11/7.5 kW (15/10 HP) <25%ED/cont> Turret: 12-station bolt-tightened turret for NL holders Rotary tool spindle: 10,000 min-1, 5.5/4.2 kW (7.5/5.6 HP) <25%ED/100%ED> Travel of Y-axis: ±50 mm (±2 in.) |
| --- | --- |
| I-003623* | Control M730UM with CELOS |
| I-003261* | CELOS - ERGOline Touch to facilitate machine operation incl. 21.5 " ERGOline Touch ® control with multi touch screens Uniform management, documentation and visualization of order, process - and machine data Networkable with CAD / CAM User friendly and productive MAPPS system |

Cooling media / chip removal

| I-003486 | (SY/700)Chip conveyor (right discharge, hinge type) (/700) Chips are loaded onto hinge type plate and discharged to outside of the machine. Effective in discharging long chips. Not suitable to discharge minute powdery chips generated when machining cast or gun metal etc because chips may flow into coolant tank. Suitable for conveying chips such as steel(long, short), aluminum(long), SUS(long, short), brass(long), copper(long). - 2-axis turning - Chip conveyor capacity: 370 L/h - Coolant throughput: Max. 100 L/min - MC, Y, S, SMC, SY - Chip conveyor capacity: 470 L/h - Coolant throughput: Max. 240 L/min - 2-axis turning: NP-00210.R1 (Enomoto BeA) - MC, Y, S, SMC, SY: NP-00063.R6 (Enomoto BeA) |
| --- | --- |
| I-003480 | (SY/700)High-pressure coolant system (635/ 1,040 W) With this specification, abilities to remove the chips and to cool the tool or workpiece during cutting are higher compared. |

Clamping device

| I-003492 | (SY/700)(Spindle 1) KITAGAWA 10"" Hollow Chuck Unit BB210A821 + KITAGAWA Hollow SR1781C21 |
| --- | --- |
| I-003493 Revolver | (SY/700)(Spindle 2) KITAGAWA 6" Hollow Chuck Unit B206A521F + HOWA Solid C1SB115M4774 |
| I-003483 | (SY/700)Overhang of O.D. cutting rotary tool: 100 mm (limited Y-axis travel) (Y, SY,SMC) |

Tailstock

I-003188 	Spindle 2 tailstock specifications

Various options

| I-003482 | (SY/700)Chuck foot switch (double) for spindle 1, 2 |
| --- | --- |
| I-004166 | Signal tower 4 layers (Red, yellow, green, blue) |
| I-003481 | (SY/700)Bar feeder I/F (LNS) (multiple) |
| I-EU0003 | Voltage transformer 45 kVA three phase autotransformer in cabinet CLPB 26F- 0727T04001 |
| I-002750 | Black specification |

Special constructions

| SK001 |  | I-008290 Multi counter display |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ___________ |